| Economic Development, Investment, Trade & Natural Resources |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Petroleum Branch |  |  |  |  |  |  |  |  |
| 360-1395 Ellice Ave, Winnipeg, MB R3G 3P2 |  |  |  |  |  |  |  |  |
| T 204-945-6577 F 204-945-0586 |  |  |  |  |  |  |  |  |
| www.manitoba.ca |  |  |  |  |  |  |  |  |
| WEEKLY WELL ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| PETROLEUM INDUSTRY ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| (January 1/25 -- February 03/25 ) |  |  |  |  |  |  |  |  |
| DRILLING ACTIVITY |  |  | To Feb 03/25 |  |  | To Feb 05/24 |  | 2024 Total |
| Drilling Licences Issued |  |  | 11 |  |  | 15 |  | 195 |
| Licences Cancelled |  |  | 2 |  |  | 4 |  | 13 |
| Vertical Wells Drilled |  |  | 0 |  |  | 0 |  | 3 |
| Horizontal Wells Drilled |  |  | 22 |  |  | 19 |  | 194 |
| Stratigraphic Test Holes Drilled |  |  | 0 |  |  | 0 |  | 0 |
| Wells Drilled - Total |  |  | 22 |  |  | 19 |  | 197 |
| No. of Metres Drilled |  | 49638 |  |  | 46004 |  |  | 471579 |
| Wells Re-entered |  |  | 1 |  |  | 1 |  | 1 |
| Wells Being Drilled |  |  | 2 |  |  | 3 |  | 0 |
| No. of Active Rigs |  |  | 2 |  |  | 3 |  | 0 |
| Wells Licenced but Not Spudded |  |  | 40 |  |  | 52 |  | 54 |
| Wells Completed as Potential Oil Wells |  |  | 22 |  |  | 19 |  | 195 |
| Wells Abandoned Dry |  |  | 0 |  |  | 1 |  | 2 |
| Wells Drilled but Not Completed |  |  | 0 |  |  | 0 |  | 0 |
| Other Completions |  |  | 0 |  |  | 0 |  | 1 |
| New Wells on Production |  |  | 12 |  |  | 0 |  | 0 |
| GEOPHYSICAL ACTIVITY |  |  |  |  |  |  |  |  |
| Geophysical Programs Licenced |  |  | 0 |  |  | 0 |  | 2 |
| Licences Cancelled |  |  | 0 |  |  | 0 |  | 0 |
| Kilometers Licenced |  |  | 0.00 |  |  | 0.00 |  | 50.31 |
| Kilometers Run |  |  | 0.00 |  |  | 0.00 |  | 50.31 |
|  |  |  | 2024/2025 |  |  | 2023/2024 |  |  |
| OIL PRICES (Average) |  | $/M3 ¦ $/BBL |  |  | $/M3 ¦ $/BBL |  |  |  |
| Month of December |  | $579.60 | $92.10 |  | $500.81 | $79.58 |  |  |
| Month of January |  | $604.40 | $96.04 |  | $535.65 | $85.11 |  |  |
| OIL PRODUCTION (M3) |  |  | 2025 |  |  | 2024 |  |  |
| Month of January |  | 200,652.1 |  | M3 | 187,461.4 |  | M3 |  |

WEEKLY WELL ACTIVITY REPORT

03 February 2025

| Lic. No.: 12192 | Corex Virden HZNTL 8-3-11-26 (WPM) UWI: 102.08-03-011-26W1.00 - Leg #1 Status: Completing (COMP) Completing: 26-Jan-2025 |
| --- | --- |
| Lic. No.: 12196 | Tundra Daly Sinclair HZNTL B13-21-9-29 (WPM) UWI: 104.13-21-009-29W1.00 - Leg #1 Spud Date: 29-Jan-2025 K.B. Elevation: 536.40 m Finished Drilling: 02-Feb-2025 Total Depth: 2334.00 m Rig Released: 03-Feb-2025 Surface Casing: 244.50 mm @ 136.00 m with 7.0 t Production Casing: 139.7mm @ 2334 m with 28 t Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 03-Feb-2025 |
| Lic. No.: 12227 | Goodlands Unit No. 2 HZNTL C4-4-1-24 (WPM) UWI: 105.04-04-001-24W1.00 - Leg #1 K.B. Elevation: 497.60 m Finished Drilling: 25-Jan-2025 Total Depth: 2966.00 m Rig Released: 26-Jan-2025 Surface Casing: 219.10 mm @ 148.00 m with 7.0 t Production Casing: 139.7 mm @ 2966 m with 41 t Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 26-Jan-2025 |
|  | Status: Completing (COMP) Completing: 30-Jan-2025 |
| Lic. No.: 12228 | Goodlands Unit No. 3 Prov. HZNTL 4-16-1-24 (WPM) UWI: 100.04-16-001-24W1.00 - Leg #1 Spud Date: 27-Jan-2025 K.B. Elevation: 497.09 m Finished Drilling: 30-Jan-2025 Total Depth: 2285.00 m Rig Released: 31-Jan-2025 Surface Casing: 219.10 mm @ 146.00 m with 7.0 t Production Casing: Run |
|  | Status: Moving In Rotary Tools (MIRT) Moving In Rotary Tools: 27-Jan-2025 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 31-Jan-2025 |
| Lic. No.: 12233 | Daly Unit No. 3 Prov. HZNTL B4-14-10-28 (WPM) UWI: 104.04-14-010-28W1.00 - Leg #1 Spud Date: 28-Jan-2025 K.B. Elevation: 500.40 m Finished Drilling: 31-Jan-2025 Total Depth: 1936.00 m Rig Released: 01-Feb-2025 Surface Casing: 244.50 mm @ 150.00 m with 10.5 t Production Casing: 139.7 mm @ 1936 m with 37 t Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 01-Feb-2025 |
| Lic. No.: 12254 | Cromer Unit No. 4 HZNTL B5-24-9-28 (WPM) UWI: 103.05-24-009-28W1.00 - Leg #1 Finished Drilling: 26-Jan-2025 Total Depth: 2397.00 m Rig Released: 27-Jan-2025 Surface Casing: 244.50 mm @ 150.00 m with 10.5 t Production Casing: 139.7 mm @ 2397 m with 45.75 t Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 27-Jan-2025 |
| Lic. No.: 12255 | Tundra Pierson HZNTL A6-15-2-28 (WPM) UWI: 102.06-15-002-28W1.00 - Leg #1 Status: Completing (COMP) Completing: 23-Jan-2025 |
| Lic. No.: 12259 | Tundra Daly Sinclair HZNTL D16-36-9-28 (WPM) UWI: 106.16-36-009-28W1.00 - Leg #1 Status: Completing (COMP) Completing: 24-Jan-2025 |
| Lic. No.: 12260 | Daly Unit No. 3 HZNTL 5-13-10-28 (WPM) UWI: 102.05-13-010-28W1.00 - Leg #1 Spud Date: 01-Feb-2025 K.B. Elevation: 503.43 m Surface Casing: 244.50 mm @ 145.00 m with 10.5 t Status: Drilling Ahead (DR) Drilling Ahead: 03-Feb-2025 |
| Lic. No.: 12261 | Tundra Pierson HZNTL 4-17-2-28 (WPM) UWI: 100.04-17-002-28W1.00 - Leg #1 Status: Completing (COMP) Completing: 25-Jan-2025 |
| Lic. No.: 12262 | Tundra Pierson HZNTL A16-31-2-28 (WPM) UWI: 102.16-31-002-28W1.00 - Leg #1 Status: Completing (COMP) Completing: 27-Jan-2025 |
| Lic. No.: 12263 | Tundra Daly Sinclair HZNTL A16-10-10-29 (WPM) UWI: 103.16-10-010-29W1.00 - Leg #1 Status: Completing (COMP) Completing: 28-Jan-2025 |
| Lic. No.: 12264 | Tundra Daly Sinclair HZNTL B16-10-10-29 (WPM) UWI: 104.16-10-010-29W1.00 - Leg #1 Status: Completing (COMP) Completing: 31-Jan-2025 |
| Lic. No.: 12265 | Tundra Daly Sinclair Prov. HZNTL A9-5-10-29 (WPM) UWI: 102.09-05-010-29W1.00 - Leg #1 Finished Drilling: 27-Jan-2025 Total Depth: 2360.00 m Rig Released: 28-Jan-2025 Surface Casing: 244.50 mm @ 137.00 m with 7.0 t Intermediate Casing: 139.7 mm @ 2269 m with 28 t Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 28-Jan-2025 Status: Completing (COMP) Completing: 01-Feb-2025 |
| Lic. No.: 12270 | Daly Unit No. 4 HZNTL C10-35-9-28 (WPM) UWI: 106.10-35-009-28W1.00 - Leg #1 Spud Date: 30-Jan-2025 K.B. Elevation: 503.58 m Finished Drilling: 01-Feb-2025 Total Depth: 1720.00 m Rig Released: 02-Feb-2025 Surface Casing: 244.50 mm @ 151.00 m with 7.5 t Production Casing: 139.7mm @ 1720 m with 32 t Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 02-Feb-2025 |
| Lic. No.: 12271 | Tundra et al Daly Sinclair HZNTL A12-2-10-29 (WPM) UWI: 103.12-02-010-29W1.00 - Leg #1 Status: Completing (COMP) Completing: 02-Feb-2025 |
| Lic. No.: 12272 | Tundra et al Daly Sinclair HZNTL B12-2-10-29 (WPM) UWI: 104.12-02-010-29W1.00 - Leg #1 Finished Drilling: 29-Jan-2025 Total Depth: 2123.00 m Rig Released: 30-Jan-2025 Surface Casing: 244.50 mm @ 144.00 m with 7.0 t Production Casing: 139.7 mm @ 2123 m with 28 t |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 30-Jan-2025 |
| Lic. No.: 12274 | Tundra Waskada HZNTL 8-17-1-24 (WPM) UWI: 102.08-17-001-24W1.00 - Leg #1 Spud Date: 31-Jan-2025 K.B. Elevation: m Surface Casing: 219.10 mm @ 142.00 m with 7.0 t |
|  | Status: Running Casing (CSG) Running Casing: 03-Feb-2025 |
| Lic. No.: 12280 | Tundra Pierson HZNTL 13-2-3-29 (WPM) UWI: 103.13-02-003-29W1.00 - Leg #1 Status: Completing (COMP) Completing: 25-Jan-2025 |
| Lic. No.: 12285 | Tundra Daly Sinclair HZNTL A4-31-9-29 (WPM) UWI:102.04-31-009-29W1.00 UWI:102.04-31-009-29W1.02 UWI:102.04-31-009-29W1.03 UWI:102.04-31-009-29W1.04 UWI:100.13-30-009-29W1.05 Tundra Daly Sinclair HZNTL A04-31-09-29 (WPM) Licence Issued: 31-Jan-2025 Licensee: Tundra Mineral Rights: Tundra Contractor: Ensign - Trinidad Drilling Inc. - Rig# 9 Surface Location: 13C-29-9-29 Co-ords: 104.79 m S of N of Sec 29 99.57 m E of W of Sec 29 Grd Elev: 540.76 m Proj. TD: 2400.43 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12286 | Tundra Waskada et al HZNTL A11-14-2-25 (WPM) UWI:103.11-14-002-25W1.00 Tundra Waskada et al HZNTL A11-14-02-25 (WPM) Licence Issued: 31-Jan-2025 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 10 Surface Location: 10D-15-2-25 Co-ords: 589.42 m S of N of Sec 15 562.80 m W of E of Sec 15 Grd Elev: 473.08 m Proj. TD: 2129.90 m (Triassic) Field: Waskada Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12287 | Tundra Pierson HZNTL 5-19-1-28 (WPM) UWI:100.05-19-001-28W1.00 Tundra Pierson HZNTL 05-19-01-28 (WPM) Licence Issued: 31-Jan-2025 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 10 Surface Location: 5C-20-1-28 Co-ords: 640.00 m N of S of Sec 20 60.00 m E of W of Sec 20 Grd Elev: 458.52 m Proj. TD: 2508.10 m (Triassic) Field: Pierson Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12288 | Daly Unit No. 1 Prov. HZNTL C15-4-10-28 (WPM) UWI:106.15-04-010-28W1.00 Daly Unit No. 1 Prov. HZNTL C15-04-10-28 (WPM) Licence Issued: 31-Jan-2025 Licensee: Corex Resources Ltd Mineral Rights: Corex Resources Ltd Contractor: Ensign Drilling Inc. - Rig# 14 Surface Location: 14A-5-10-28 Co-ords: 340.00 m S of N of Sec 5 669.81 m E of W of Sec 5 Grd Elev: 512.63 m Proj. TD: 2631.98 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12289 | Tundra et al Daly Sinclair HZNTL C5-5-9-28 (WPM) UWI:104.05-05-009-28W1.00 Tundra et al Daly Sinclair HZNTL C05-05-09-28 (WPM) Licence Issued: 03-Feb-2025 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 5C-4-9-28 Co-ords: 680.00 m N of S of Sec 4 66.40 m E of W of Sec 4 Grd Elev: 489.10 m Proj. TD: 2437.00 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12290 | Tundra et al Daly Sinclair HZNTL D5-5-9-28 (WPM) UWI:105.05-05-009-28W1.00 Tundra et al Daly Sinclair HZNTL D05-05-09-28 (WPM) Licence Issued: 03-Feb-2025 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 5C-4-9-28 Co-ords: 700.00 m N of S of Sec 4 66.40 m E of W of Sec 4 Grd Elev: 489.20 m Proj. TD: 2392.30 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12291 | Tundra et al Daly Sinclair HZNTL D12-5-9-28 (WPM) UWI:105.12-05-009-28W1.00 Tundra et al Daly Sinclair HZNTL D12-05-09-28 (WPM) Licence Issued: 03-Feb-2025 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 5C-4-9-28 Co-ords: 720.00 m N of S of Sec 4 66.40 m E of W of Sec 4 Grd Elev: 488.85 m Proj. TD: 2501.60 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |